Think of each row (Step) as one moment in time.

Data flow (left → right):

Step

↓

True_Temperature  →  Raw_Temperature (+ Error_Type)  →  Filtered_Temperature  →  Fan_Before / Fan_After

  Step: time index (t = 1, 2, …).

  True_Temperature: the real environment (ground truth).

  Raw_Temperature: what the sensor reports after errors are injected.

  Error_Type: how the raw value was corrupted (noise / missing / spike / normal).

  Filtered_Temperature: cleaned value after the filter (moving average).

  Fan_Before: decision using Raw_Temperature.

  Fan_After: decision using Filtered_Temperature.

At each Step t:

- Ground truth

T_true(t)

- Sensor reading with error

T_raw(t) = T_true(t) + error(t)

- Filtering (uses history)

T_filtered(t) = avg( T_raw(t), T_raw(t-1), T_raw(t-2) )

(ignores None if missing)

- Control (same rule for both)

if Temp > 26 → ON
if Temp < 24 → OFF
else → keep previous state

- Fan_Before uses T_raw(t)

- Fan_After uses T_filtered(t)

Error type:

1. normal

return value

👉 Meaning:

- sensor works correctly

- no error

📌 In real world:

- ideal condition

- baseline for comparison

🌊 2. noise

value + random.uniform(-1.5, 1.5)

👉 Meaning:

- small random disturbance

📌 In real world:

- electrical interference

- environment fluctuation

- sensor precision limit

Example:

True = 25.0  
Noise = ±1.5  
→ Raw = 23.8 ~ 26.5

❌ 3. missing

return None

👉 Meaning:

- no data

📌 In real world:

- network failure

- sensor offline

- transmission loss

⚡ 4. spike

value + (± random 3~5)

👉 Meaning:

- sudden abnormal jump

📌 In real world:

- sensor malfunction

- cyber attack (false data injection)

- hardware glitch

Example:

True = 25  
Spike → 30 or 20

🎯 2. Why define them like this?

👉 Because they match three major categories in CPS research (RQ1)

| Category | Your simulation |
| --- | --- |
| Data noise | noise |
| Missing data | missing |
| Abnormal values | spike |

✔ This is exactly what papers describe

📊 3. Why use these specific numbers?

🔹 noise: ±1.5

👉 Why small?

- noise = minor error

- should not break system completely

- but affects stability

🔹 spike: ±3 ~ 5

👉 Why large?

- spike = serious abnormal

- clearly crosses threshold

- triggers wrong decisions

🔹 missing: None

👉 Why None?

- represents no signal at all

- forces system to:

  - wait

  - or use previous state

⚖️ 4. Why use probability weights?

weights=[0.55, 0.25, 0.10, 0.10]

👉 Meaning:

| Type | Probability | Why |
| --- | --- | --- |
| normal | 55% | most data is correct |
| noise | 25% | common |
| missing | 10% | occasional |
| spike | 10% | rare but important |

🎯 Why this is important

👉 makes simulation realistic

Real world:

- mostly normal

- some noise

- few failures

🔥 5. Why this design is GOOD for your research

Because it allows you to:

✔ simulate real CPS conditions
✔ test different error impacts (RQ2)
✔ evaluate solutions (RQ3)

🧠 6. What your code is doing (simple)

error = random.choices(...)

👉 randomly choose error type

Then:

if normal → keep value  
if noise → small change  
if missing → no value  
if spike → big jump

🎯 7. How to explain in your report (copy this)

Different types of sensor errors were simulated, including normal data, noise, missing data, and spike errors. Noise represents small random fluctuations commonly observed in real sensors, while missing data simulates communication failures or sensor outages. Spike errors represent sudden abnormal values caused by faults or malicious attacks. The probability distribution of these error types was designed to reflect realistic CPS environments, where most data is normal but occasional errors occur.

🚀 8. One sentence summary (remember this)

👉 normal = correct
👉 noise = small disturbance
👉 missing = no data
👉 spike = big abnormal error

🎯 Why this is high score

Because you show:

✔ understanding of sensor error types (RQ1)
✔ mapping to real-world CPS
✔ justification of design choices